WFS software

| Prepared by |  | A. Puglisi |
| --- | --- | --- |
| Approved by |  |  |
| Released by |  |  |

ABSTRACT

This document describe the components of the WFS software.

Modification Record

| Version | Date | Author | Section/Paragraph affected | Reason/Remarks |
| --- | --- | --- | --- | --- |
|  | 25 Nov 2016 | A. Puglisi |  | Document release |

Abbreviations, acronyms and symbols

| Symbol | Description |
| --- | --- |
| LBT | Large Binocular Telesecope |
| WFS | WaveFront Sensing unit of the LBT AO system |

Contents

1.	Introduction	5

2.	Low-level hardware controllers	5

1.1.1	Power management	5

1.1.2	CCD controllers	5

1.1.3	Motor controllers	5

1.1.4	BCU controllers	5

1.1.5	Piezo controllers	6

3.	Telemetry system and auxiliary loops	6

4.	High level coordination	6

# Introduction

The WFS software is divided into a number of components in a roughly two-layers structure: low-level hardware controllers and high-level coordination scripts and interfaces.

# Low-level hardware controllers

Most low-level hardware controllers are written in C++. They all derive from the AOApp C++ class and are controlled via MsgD-RTDB variables and, in some cases, simple messages.

### Power management

- RelayCtrl – handles the main power board and the temperature reading for W unit #1

- RelayCtrlW2 – handles the main power board and the temperature reading for W unit #2

- PowerCtrl – handles the power switches on the BCU39 on W unit #1

- PicCtrl: handles the power switches on the flowerpot PIC board on W unit #1

- PicCtrlW2: handles the power switches on the flowerpot PIC board on W unit #2

### CCD controllers

- JoeCtrl: two instances control the ccd39 and ccd47.

### Motor controllers

- CopleyCtrl: three instances control the three-axes Bayside stages

- SimpleMotorCtrl:  seven instances control the two filter wheels, the two ADC motors, the pupil rerotator, the cube rotator and translation stage.

### BCU controllers

- SlopeCompCtrl: manages the slope computer BCU39

- TechViewerCtrl manages the frame grabber BCU47, which also includes the tip-tilt and camera lens waveform generator

### Piezo controllers

- TTCtrl.: controls the tip-tilt mirror via the BCU47

- CameraLensCtrl: controls the camera lens stage via the BCU47

# Telemetry system and auxiliary loops

- MasterDiagnostic:  collects realtime telemetry data from the BCUs and writes them into local shared memory

- housekeeperWFS: collects non-realtime telemetry information (e.g. temperatures) from BCUs

- OptLoopDiagnostic:  performs various housekeeping tasks (dark following loop, intensity check) and writes telemetry data on disk when requested.

- pupilcheck.py: checks the pupil position on the Pyramid CCD (ccd39)

- ccd47read.py: continuously reads the ccd47 and sends the image to the AOS

- DBFiller: collects data for Zabbix telemetry system

- TrueSense: performs truth-sensing: collects data from ccd39 and calculates current low-order aberrations.

- gopt.py : realtime optical gain calculation and correction

# High level coordination

- WfsArbitrator:  state machine controlling the overall WFS state